VECTAB.DOC  2/8/95

This file documents the functions accessible via the jump table

located in the ROM at address 0x0030.

This document is unlikely to be relevant unless you are using a

non-HiTech C compiler, e.g. IAR.

Functions called "undoc" are not documented for customer use.  Some

of them, used incorrectly, could cause corruption of the EEPROM and

make the KD485 useless.  Others are simply for internal KK Systems

use and their operation might vary.

Each "jmp" is the full H8 four-byte jump instruction.

Each function should be assumed to corrupt all registers R0,R1,R2,R3

except of course where a register returns a parameter.  R4-R7 are

preserved.  It should also be assumed that all functions enable

interrupts, so if you call it from a piece of code which has

interrupts disabled, they will get re-enabled.

org 0x0030

jmp undoc

jmp undoc

jmp undoc

jmp undoc

jmp undoc

jmp _atof

jmp _atoi

jmp _atol

jmp _chk_par_cs

jmp _drvctrl

jmp _exit

jmp _fgetc

jmp _fgetcnd

jmp _fputc

jmp _freadc

jmp undoc

jmp _fwritec

jmp _geterrs

jmp _getportcfg

jmp _getrts

jmp _getsw

jmp _getvecbase

jmp _ilfprintf

jmp _ilsprintf

jmp _ilvfprintf

jmp _ilvsprintf

jmp undoc

jmp _ipqclear

jmp _ipqcount

jmp _isalpha

jmp _isdigit

jmp _isspace

jmp undoc

jmp _loadtimer

jmp _nvwrblk

jmp _nvwrblku

jmp _opqcount

jmp _opqspace

jmp undoc

jmp _readtimer

jmp _setportcfg

jmp _sscanf

jmp _strcmp

jmp _strlen

jmp _strncmp

jmp _sysdata

jmp _testset

jmp _testsetw

jmp _tolower

jmp _toupper

jmp _uled

jmp _vsscanf

jmp _waggle_pin

jmp _wait_485

jmp _wdg_disable

jmp _wdg_enable

jmp _wdg_mode

jmp _wdg_pulse

; **********************************************************************

; Standard C

; R3 -> numeric string, result in ??

_atof

; **********************************************************************

; Standard C

; R3 -> numeric string, result in R0

_atoi

; **********************************************************************

; Standard C

; R3 -> numeric string, result in R0R1

_atol

; **********************************************************************

; Check checksum of parallel EEPROM. A 16-bit checksum, computed over WORDS.

; If R3=0, returns diff between the two in R0.

; If R3=1, returns 'what it should be' in R0.

; Assumes that program length word is EVEN!

_chk_par_cs

; **********************************************************************

; Function called by C via table in cext.kd4.

; Enter with:	R3 = port (1..)

; 		R2 = 0 to disable, 1 to enable

; Returns:	nothing

_drvctrl

; **********************************************************************

; This is also the C exit() function entry point.  Return to Executive.

; It reloads SP, so it can be called, or jumped to!  The Executive never

; returns anyway.

_exit

; **********************************************************************

; Receive a char from specified channel.

; Enter:	R3  = channel (1..)

; Returns:	R0L = char

;		R0L = -1 if invalid port#

_fgetc

; **********************************************************************

; A non-destructive version of fgetc.

; Enter:	R3  = channel (1..)

; Returns:	R0  = char (R0H=0)

; 		R0  = -1 if queue empty

;		R0  = -2 if invalid port #

_fgetcnd

; **********************************************************************

; Transmit a char to specified channel.

; Enter:	R3  = channel (1..)

;		R2L = char

; Returns:	R0  = 1 if port# wrong, =0 otherwise

_fputc

; **********************************************************************

; Receive a binary string from a specified channel.

; Enter:	R3 = channel (1..)

;		R2 = address of buffer

;	     	R1 = # of bytes to read

; Returns:	R0 = 1 if port# wrong, =0 otherwise

_freadc

; **********************************************************************

; Transmit a binary string to a specified channel.

; Enter:	R3 = channel (1..)

;		R2 = address of string

;	     	R1 = length of string

; Returns:	R0 = 1 if port# wrong, =0 otherwise

_fwritec

; **********************************************************************

; Return address (in R0) of the three error flag bytes

; mathserr

; p1_flags

; p2_flags

_geterrs

; **********************************************************************

; Same as setportcfg but reads-back the port config into the structure.

;

; Enter with:	R3 -> structure.

; Returns:	R0=0 if no errors, -1 otherwise

_getportcfg

; **********************************************************************

; This routine reads the RTS input.

; Enter with:	nothing

; Returns:	R0=0 if RTS inactive, R0=1 otherwise.

_getrts

; **********************************************************************

; This routine reads any switches into the structure below:

; name 	type	offset	values

; SW1  	int	  0	0,1	= off,on (ON = switch pressed)

; SW2  	int	  2	0,1	= unimplemented on KD485

; SW3  	int	  4	0,1	= unimplemented on KD485

; Enter with:	R3 -> structure.

; Returns:	nothing

_getsw

; **********************************************************************

; Return the address of ram_vectab, in R0.

; This is the int vector table (in RAM), whose contents must be modified

; if a user program wishes to hook-up an interrupt.

_getvecbase

; **********************************************************************

; Standard C

; Non-float fprintf.

; Special parameter passing.

_ilfprintf

; **********************************************************************

; Standard C

; Non-float sprintf.

; Special parameter passing.

_ilsprintf

; **********************************************************************

; Standard C

; Non-float vfprintf.

; Special parameter passing.

_ilvfprintf

; **********************************************************************

; Standard C

; Non-float vsprintf.

; Special parameter passing.

_ilvsprintf

; **********************************************************************

; This routine clears all data in a specified input queue.

; Enter with:	R3 = port # (1..)

; Returns:	R0 = 1 if port invalid, 0 otherwise

_ipqclear

; **********************************************************************

; This routine returns the number of bytes in a specified input queue.

; Enter with:	R3 = port # (1..)

; Returns:	R0 = # of bytes in the queue, -1 if port# invalid.

_ipqcount

; **********************************************************************

; Functions which were originally done via the ctype look-up table; these

; are done in code, because they are also called by some ROM functions.

; Entry in R3L, return value in R0.

_isalpha

_isdigit

_isspace

; **********************************************************************

; This routine loads a value (in ms) into one of the KD485 timers.  The timer

; is decremented (only if nonzero) every 1ms by the 1kHz timer interrupt.

; Enter with:	R3 = timer # (0..7)

;		R2 = value to load (0..65535)

; Returns:	R0 = -1 if timer # invalid, 0 otherwise

_loadtimer

; **********************************************************************

; WRITE A BLOCK TO EEPROM

; This routine writes a block to the 2000-byte 'user' non-volatile area.

; The write is verified afterwards.  Uses fast page mode; exec time approx

; 6ms per 64-byte page or part thereof.

;

; The max WR limit is 10,000-100,000 writes per byte, and NO checking is done

; to prevent repeated writes of the SAME data.

; Users should not use this function until their program is well debugged.

;

; Enter with:  	R3 = src addr in RAM (physical)

; 		R2 = dest addr within EEPROM (0..1999)

; 	     	R1 = block size (1..2000)

; Returns:	R0L=0 if OK

;		R0L=1 if parms invalid

;		R0L=2 if write failed.

;		R0L=3 if power fail signal active.

;		R0L=4 if verify error.

_nvwrblk

; **********************************************************************

; WRITE A BLOCK TO EEPROM (UNRESTRICTED)

; Enter with:  	R3 = src addr in RAM (physical)

; 		R2 = dest addr within EEPROM (0..PAR_EE_SIZE-1)

; 	     	R1 = block size (1..PAR_EE_SIZE)

; Returns:	as nvwrblk

_nvwrblku

; **********************************************************************

; This routine returns the number of bytes of data in a specified o/p queue.

; It does *not properly resolve the contents of the UART shift register. We

; just assume that there are 2 bytes in there if it is not empty.  The

; '1 byte' case cannot be detected.

;

; If you want to e.g. turn-off a RS485 driver right after the end of the last

; char of a message, loop on opqcount until it returns zero, then wait for 2

; character periods with a timer.  The function wait_485 does this.

;

; Enter with:	R3 = port # (1..)

; Returns:	R0 = # of bytes in the queue, -1 if port# invalid.

_opqcount

; **********************************************************************

; This routine returns the number of bytes of free space in a specified o/p

; queue.

;

; Enter with:	R3 = port # (1..)

; Returns:	R0 = # of bytes of space in the queue, -1 if port # invalid

_opqspace

; **********************************************************************

; This routine reads one of the KD485 timers, and returns remaining time

; (in ms).  The timer is decremented (only if nonzero) every 1ms by the 1kHz

; timer interrupt.

;

; Enter with:	R3 = timer # (0..7)

; Returns:	R0 = timer value (0..65535)

; 		R0 = 65535 if invalid timer # specified

_readtimer

; **********************************************************************

; This routine initialises a port from the structure below:

; name		type	offset	valid values

;

; kd4_port	int	  0	1,2 	= p1,2

; baud_rate	int	  2	0,1.....20 (see index table below)

; bits_word	int	  4	2,3 	= 7,8 b/word

; parity   	int	  6	0,1,2	= none,even,odd parity

; stop_bits	int	  8	0,1	= 1,2 stop bits

; rx_rts_hs	int	 10	unimplemented - no h/w h/s on KD485

; rx_dtr_hs	int	 12	unimplemented - no h/w h/s on KD485

; rx_x_hs  	int	 14	0,1	= off,on

; tx_cts_hs	int	 16	unimplemented - no h/w h/s on KD485

; tx_dsr_hs	int	 18	unimplemented - no h/w h/s on KD485

; tx_x_hs  	int	 20 	0,1	= off,on

;

; index	 baudrate

;   0	    30

;   1	    37.5

;   2	    50

;   3	    75

;   4	   100

;   5	   110

;   6	   134.5

;   7	   150

;   8	   300

;   9	   600

;  10	  1200

;  11	  2000

;  12	  2400

;  13	  3600

;  14	  4800

;  15	  7200

;  16	  9600

;  17	 19200

;  18	 38400

;  19	 57600

;  20	115200

; Enter with:	R3 -> structure.

; Returns:	R0 = 0 if no errors, -1 otherwise

; 		All input parms are range-checked.

_setportcfg

; **********************************************************************

; Standard C

; Special parameter passing.

_sscanf

; **********************************************************************

; Standard C

; Enter:	R3  -> 1st string

;		R2  -> 2nd string

; Returns:	R0  =  result

_strcmp

; **********************************************************************

; Standard C

; Enter:	R3  -> string

; Returns:	R0  =  result

_strlen

; **********************************************************************

; Standard C

; Enter:	R3  -> 1st string

;		R2  -> 2nd string

;		R1  =  length

; Returns:	R0  =  result

_strncmp

; **********************************************************************

; Fill user's buffer (@R3) with information about the KD485.

; Returns # of bytes filled-in in R0.

; If R3=0, returns # of bytes filled-in in R0 but does not fill anything.

_sysdata

; **********************************************************************

; Implement a TAS operation on byte @ R3.

; Return result (0|1) in R0L, 0 if failed, 1 if success.

_testset

; **********************************************************************

; Wait until semaphore @R3 is clear

_testsetw

; **********************************************************************

; Standard C

; Entry in R3L, return value in R0L.

_tolower

; **********************************************************************

; Standard C

; Entry in R3L, return value in R0L.

_toupper

; **********************************************************************

; This routine switches the LED on/off.

; Enter with:	R3 = 0|1 for off|on

; Returns:	nothing

_uled

; **********************************************************************

; Standard C

; Special parameter passing.

_vsscanf

; **********************************************************************

; This routine generates a very short +ve pulse on P45 pin on the CPU.

; This pin comes out on a 1mm hole in the CPU board, so it can be probed.

; This function can be called regardless of interrupt enable state.

; Enter with:	nothing

; Returns:	nothing

_waggle_pin

; **********************************************************************

; Wait for a baudrate-dependent time  and then turn on/off RS485 driver.

; This function serves two purposes:

;

; 1. Wait a baud-rate-dependent delay, then turn OFF the driver.

;    It is called at the point where opqcount(port) has just gone to zero.

;    It waits for just the right time, then turns off the driver.

; 2. Wait a baud-rate-dependent delay, then turn ON the driver.

;    This mode is useful in a RS485 slave, for achieving a delay before

;    returning a response, to make sure it is not returned too early.

;

; The exact turn-off point is not very precise, partly because of the

; 1ms timer resolution, and partly because we may not get here *immediately

; when P2 opqcount reaches zero.  Therefore, at >=9600 baud, the wait is done

; using a hardware timer which we load and then poll until it overflows.

; This special time is also user-adjustable in KDCFG.EXE, for each of the

; higher baud rates.

; But this might mean that the delay is less than the 2-3 char periods which

; is ideally required in the 'turn-ON' mode.

;

; Enter with 	r3 = port (1..)

;		r2 = desired driver state after the delay: 0=off, 1=on

_wait_485

; **********************************************************************

; Disable watchdog.

; Don't call this function unless you have a good reason to; if your

; program crashes, you may not be able to enter the Executive by pressing

; the button!  However, holding the button at _power-up_ should always

; work.

_wdg_disable

; **********************************************************************

; Enable watchdog

_wdg_enable

; **********************************************************************

; Set watchdog mode

; This routine selects whether the user's program OR the KD485 system kicks

; the watchdog timer.  At power-up it is done by the system.

;

; Enter with:	R3 = 0 for system trigger

; 		R3 = 1 for user   trigger

; Returns:	nothing

_wdg_mode

; **********************************************************************

; This pulses the watchdog, thus extending the timeout by another 1.6s.

_wdg_pulse